Tables 4 & 5 present the parameters and variables associated with the proposed model.

Table4. The model set and parameter

| Set/ Parameter | Definition |
| --- | --- |
| I | Items set |
| P | Palettes set |
| K | Container set |
| Vpp | Pallet volume p |
| Vi | Item volume i |
| Vkk | Container volume k |
| D | Total number of items to be packaged |
| (li, wi, hi) | Length, width, and height of item i |
| (lpp, wpp, hpp) | Length, width, and height of pallet p |
| M | Very large amount |
| (LKk, WKk, HKk) | Length, width, and height of container k |

Table5. The model variables

| Variable | Definition |
| --- | --- |
| 𝑋𝑖𝑝 | If item i is assigned to palette p, the value is 1, otherwise the value is 0. |
| Ypk | If pallet p is assigned to container k, the value is 1, otherwise the value is 0. |
| Up | If palette p is used, the value is 1, otherwise 0. |
| Xij- | If item i is located before item j along the length of the pallet, the value is 1, otherwise the value is 0. |
| Xij+ | If item i is located after item j along the length of the pallet, the value is 1, otherwise the value is 0. |
| Yij- | If item i is located before item j along the width of the pallet, the value is 1, otherwise the value is 0. |
| Yij+ | If item i is located after item j along the width of the pallet, the value is 1, otherwise the value is 0. |
| Zij- | If item i is located below item j in the pallet height direction, the value is 1, otherwise the value is 0. |
| Zij+ | If item i is located above item j in the pallet height direction, the value is 1, otherwise the value is 0. |
| Sij | If items i and j are assigned to the same palette, the value is 1, otherwise the value is 0. |
| Tpf | If palettes p and f are assigned to the same container, the value is 1, otherwise the value is 0. |
| (Cix, Ciy, Ciz) | Coordinates of the front-bottom-left corner point of item i |
| (Cpx, Cpy, Cpz) | Coordinates of the front-bottom-left corner point of pallet p |
| Uk | If container k is used, the value is 1, otherwise 0. |
| Apf- | If pallet p is located before pallet f along the length of the container, the value is 1, otherwise the value is 0. |
| Apf+ | If pallet p is located after pallet f along the length of the container, the value is 1, otherwise the value is 0. |
| Bpf- | If pallet p is located before pallet f along the width of the container, the value is 1, otherwise the value is 0. |
| Bpf+ | If pallet p is located after pallet f along the width of the container, the value is 1, otherwise the value is 0. |
| Cpf- | If pallet p is located below pallet f in the container height direction, the value is 1, otherwise the value is 0. |
| Cpf+ | If pallet p is located above pallet f in the container height direction, the value is 1, otherwise the value is 0. |

Hybrid model:

Objective function

Z=min(k(LKk*WKk*HKk)*Uk)                                        (1)

Minimizing the number of containers needed;

Constraints

S.t.

pXip=1                                                                                                                                               ∀ i∈I    (2)

ipXip=D                                                                                                                                                         (3)

Sij≤pXip                                                                                                                           ∀ i,j∈I , i≠j     (4)

Sij≤pXjp                                                                                                                            ∀ i,j∈I , i≠j     (5)

Sij≥ Xip+Xjp-1                                                                                                   ∀ i,j∈I , i≠j, p∈P    (6)

Sij+ Xip+Xjf≤2                                                                              ∀ i,j∈I , i≠j, p,f∈P, p≠f       (7)

pYpk≥0                                                                                                                                              ∀k∈K     (8)

kYpk=Up                                                                                                                                              ∀p∈P   (9)

Ypk≤Uk                                                                                                                                                  ∀p∈P    (10)

Cix+li≤lpp.Up +1-XipM                                                                                             ∀ i∈I, p∈P    (11)

Ciy+wi≤wpp.Up +1-XipM                                                                                        ∀ i∈I, p∈P     (12)

Ciz+hi≤hpp.Up +1-XipM                                                                                           ∀ i∈I, p∈P    (13)

Cix+li≤Cjx+1-Xij-M                                                                                                  ∀ i,j∈I, i≠j    (14)

Cjx+lj≤Cix+1-Xij+M                                                                                                  ∀ i,j∈I, i≠j    (15)

Ciy+wi≤Cjy+1-Yij-M                                                                                                ∀ i,j∈I, i≠j    (16)

Cjy+wj≤Ciy+1-Yij+M                                                                                                 ∀ i,j∈I, i≠j   (17)

Ciz+hi≤Cjz+1-Zij-M                                                                                                  ∀ i,j∈I, i≠j   (18)

Cjz+hj≤Ciz+1-Zij+M                                                                                                  ∀ i,j∈I, i≠j   (19)

Xij-+Xij++Yij-+Yij++Zij-+Zij+≥Sij                                                                      ∀ i,j∈I, i≠j     (20)

Xij-+Xij+≤Sij                                                                                                               ∀ i,j∈I, i≠j      (21)

Yij-+Yij+≤Sij                                                                                                               ∀ i,j∈I, i≠j      (22)

Zij-+Zij+≤Sij                                                                                                               ∀ i,j∈I, i≠j      (23)

iVi.Xip≤Vpp                                                                                                                        ∀  p∈P      (24)

Cpx+lpp≤LKk+1-YpkM                                                                                   ∀ p∈P , k∈K     (25)

Cpy+wpp≤WKk+1-YpkM                                                                             ∀   p∈P , k∈K     (26)

Cpz+hpp≤HKk+1-YpkM                                                                              ∀  p∈P , k∈K       (27)

Cpx+lpp≤Cfx+1-Apf-M                                                                            ∀  p,f∈P , p≠f    (28)

Cfx+lfp≤Cpx+1-Apf+M                                                                             ∀ p,f∈P , p≠f    (29)

(Cpy+wpp)≤Cfy+1-Bpf-M                                                                           ∀ p,f∈P , p≠f    (30)

(Cfy+wfp)≤Cpy+1-Bpf+M                                                                            ∀ p,f∈P , p≠f   (31)

(Cpz+hpp)≤Cfz+1-Cpf-M                                                                            ∀ p,f∈P , p≠f      (32)

(Cfz+hfp)≤Cpz+1-Cpf+M                                                                            ∀  p,f∈P , p≠f      (33)

Tpf≤kYpk                                                                                                                       ∀ p,f∈P , p≠f    (34)

Tpf≤kYfk                                                                                                                         ∀ p,f∈P , p≠f  (35)

Tpf≥ Ypk+Yfk-1                                                                                             ∀ p,f∈P , p≠f, k∈K   (36)

Tpf+ Ypk+Yfr≤2                                                                 ∀ p,f∈P , p≠f, k∈K, r∈K, k≠r      (37)

Apf-+Apf++Bpf-+Bpf++Cpf-+Cpf+≥Tpf                                                      ∀  p,f∈P , p≠f     (38)

Apf-+Apf+≤ Tpf                                                                                                             ∀  p,f∈P , p≠f  (39)

Bpf-+Bpf+≤Tpf                                                                                                       ∀  p,f∈P , p≠f   (40)

Cpf-+Cpf+≤Tpf                                                                                                        ∀  p,f∈P , p≠f   (41)

pVpp.Ypk≤Vkk                                                                                                                ∀k∈K                    (42)              Cix, Ciy, Ciz, Cpx, Cpy, Cpz≥0                                                                                                                                                        (43)

Xip,Up,Ypk,Xij- Xij+, Yij-, Yij+, Zij-, Zij+, Sij, Tpf, Uk, Apf-, Apf+, Bpf-, Bpf+, Cpf-, Cpf+∈0,1                            (44)

Constraints 2 to 10 ensure organized and efficient use of space during transportation. According to constraint 2 each item shall be packed onto only one pallet and constraint 3 ensure All goods shall be packaged. Two specific items assigned to a pallet shall not be assigned to another pallet (constraints 4-7). Also every pallet shall be designated for a container exclusively when both the pallet and the container are utilized (constraints 8&9) and every pallet shall be assigned to at most one container (constraint 10). The following constraints are crucial for maintaining the integrity of the packaging: The items packaged onto a pallet shall not exceed the dimensions of the container (constraints 11-13). The items packaged onto a pallet shall not overlap (constraints 14-19) and the spatial arrangement of packaged items relative to one another on a pallet must be considered (constraints 20-23). Constraint 24 states that The total volume of items packaged on a pallet shall not exceed the total volume of that pallet.

Additionally, similar principles apply to pallets within containers: The size of pallets loaded into a container must not exceed the container’s dimensions (constraints 25-27), the pallets loaded into a container must not overlap (constraints 28-33) and the spatial arrangement of pallets relative to one another in a container must be maintained (constraints 38-41).

Lastly, the following constraints ensure effective logistics management: According to constraint 42 the total volume of pallets loaded into a container must not exceed the container's volume. Also two pallets assigned to a specific container cannot be assigned to another container (constraints 34-37).